Fixed Point Implementation of a

Single Side Band – Lower Side Band

Communication System

Calvin Kerner and Todd Goldfinger

ECE 497 – DSP System Design

Dr. Wayne Padgett

Rose-Hulman Institute of Technology

11/07/2003

Design System 1

Transmitter

# The input is upsampled from 8000 hz to 96000 hz.  Then it is lowpass filtered with an elliptic filter to remove images.  An elliptic filter was chosen because it was relatively low order and in previous simulations it produced acceptable output.  It is also converted to SOS for stability and to reduce roundoff error.  Linfinity scaling is performed between each section to meet the requirement of no full scale sinusoid overflow.  Next, the signal is put through an FIR Hilbert transformer to make the signal SSB.  Then it is modulated into one of four channels: 30khz, 35khz, 40khz, 45khz.  The signal is normalized to a maximum of .99999 for testing purposes.  This ends the transmission stage.

Table 1

| Upsample Image Removal (Elliptic) |  |  | Quantized |  |
| --- | --- | --- | --- | --- |
| Order | 6 |  | Type | DF2T |
| Pass ripple (dB) | 3 |  | coefficient bits | Q2.14 |
| Passband stop (hz) | 4000 |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Stopband start (hz) | minimal |  |  |  |
| Hilbert Transform (Optimal Equiripple FIR) |  |  | Quantized |  |
| order | 100 |  | Type | FIR |
|  |  |  | coefficient bits | Q1.15 |
|  |  |  | product bits | Q2.30 |
|  |  |  | sum bits | Q2.30 |

## Receiver

# The received signal is bandpass filtered using a type II chebychev filter.  The reasons for using chebychev and for using bandpass are explained in the comparison section.  This filter is also converted to SOS and uses Linfinity scaling.  The signal is demodulated back to baseband and lowpass filtered with a type II chebychev filter to remove aliased images.  It is SOS with Linfinity scaling.  The signal is downsampled to recover the approximate original signal.  Although both the filters in the receiver are calculated at runtime to be minimal order, in a real system the filters could be calculated ahead of time and stored in a lookup table.

Table 2

| Bandpass Channel Select (Chebychev2) |  |  | Quantized |  |
| --- | --- | --- | --- | --- |
| Order | minimal |  | Type | DF2T |
| Pass ripple (dB) | 5 |  | coefficient bits | Q2.14 |
| Passband range (hz) | 4000 |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Transition range (hz) | 1000 |  |  |  |
| Lowpass Anti-alias (Chebychev2) |  |  | Quantized |  |
| Order | minimal |  | Type | DF2T |
| Pass ripple (dB) | 5 |  | coefficient bits | Q2.14 |
| Passband stop (hz) | 4000 |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Stopband start (hz) | minimal |  |  |  |

Design System 2

Transmitter

# The input is upsampled from 8000 hz to 96000 hz.  Then it is lowpass filtered with an elliptic filter to remove images.  An elliptic filter was chosen because it was relatively low order and in previous simulations it produced acceptable output.  It is also converted to SOS for stability and to reduce roundoff error.  Linfinity scaling is performed between each section to meet the requirement of no full scale sinusoid overflow.  Next, the signal is put through an FIR Hilbert transformer to make the signal SSB.  Then it is modulated into one of four channels: 30khz, 35khz, 40khz, 45khz.  The signal is normalized to a maximum of .99 for testing purposes.  This ends the transmission stage.

Table 3

| Upsample Image Removal (Elliptic) |  |  | Quantized |  |
| --- | --- | --- | --- | --- |
| Order | 6 |  | Type | DF2T |
| Pass ripple (dB) | 3 |  | coefficient bits | Q2.14 |
| Passband stop (hz) | 4000 |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Stopband start (hz) | minimal |  |  |  |
| Hilbert Transform (Optimal Equiripple FIR) |  |  | Quantized |  |
| order | 100 |  | Type | FIR |
|  |  |  | coefficient bits | Q2.14 |
|  |  |  | product bits | Q2.30 |
|  |  |  | sum bits | Q2.30 |

## Receiver

# The received signal is lowpass filtered using an elliptic filter.  The reasons for using elliptic filters are explained in the comparison section.  This filter is also converted to SOS and uses Linfinity scaling.  The signal is demodulated back to baseband and lowpass filtered with another elliptic filter to remove aliased images.  It is also SOS with Linfinity scaling.  The signal is downsampled to recover the approximate original signal.

Table 4

| Lowpass Channel Select (Elliptic) |  |  | Quantized |  |
| --- | --- | --- | --- | --- |
| Order | 6 |  | Type | DF2T |
| Pass ripple (dB) | 3 |  | coefficient bits | Q2.14 |
| Passband range (hz) | 30k-45k |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Transition range (hz) | 600 |  |  |  |
| Lowpass Anti-alias (Elliptic) |  |  | Quantized |  |
| Order | 6 |  | Type | DF2T |
| Pass ripple (dB) | 3 |  | coefficient bits | Q2.14 |
| Passband stop (hz) | 4000 |  | product bits | Q2.30 |
| Stopband attenuation (dB) | 40 |  | sum bits | Q2.30 |
| Stopband start (hz) | minimal |  |  |  |

## Design Comparisons

Although there are several differences between System 1 and System 2, only the filter types will be discussed.  System 1 uses type 2 chebychev filters in the receiver.  The reason for using chebychev is that the pass band is flat except at the edges.  This means possibly less signal distortion for a given SNR.  Alternatively, the distortion can be increased allowing an increase in SNR or a lower order filter.  Unfortunately, there wasn’t time to experiment with this.  Also, for Chebychev filters it turns out the poles are closer to the origin than elliptic filters, shrinking the impulse response and raising SNR (see Figures 1 and 2). System 2 uses elliptical filters.  These will be slightly lower order, but the passband will have ripple.  The other difference in the filters is that System 1 uses a bandpass filter and System 2 uses only lowpass filters.  The obvious benefit of lowpass filters is a lower order filter.  The reason for trying bandpass filtering is that by bandpass filtering first, the area outside the channel range is attenuated twice, thereby making the SNR higher.  Or, for a given SNR, the stopband attenuation could be set higher allowing a lower order filter.  Again, there wasn’t time to experiment with this.  Also bandpass filtering first allows the use of a lower order lowpass filter when doing anti-aliasing.  Any other differences in filters are due to basic parameters like ripple, attenuation, and cutoff choices.  These choices can be seen in the tables.

An obvious improvement on the system would be to go to all FIR filtering with polyphase.  Other improvements such as tweaking filter cutoff values and optimizing distortion versus filter order can also be done.

## System Comparisons

Table 5

| * = Less than 10 overflows in each individual channel | Reference | System 1 | System 2 |
| --- | --- | --- | --- |
| sweeptestNovr | 0 | 0 | 0 |
| chanSNR30 (dB) | 24.6556 | 24.4107 | 24.5339 |
| chanSNR35 (dB) | 20.5704 | 22.1514 | 22.4564 |
| chanSNR40 (dB) | 24.6604 | 24.9089 | 24.9072 |
| chanSNR45 (dB) | 25.472 | 24.8992 | 25.3199 |
| SIR30 (dB) | 20.3081 | 39.0913 | 28.7385 |
| SIR35 (dB) | 22.2472 | 37.6002 | 30.0177 |
| SIR40 (dB) | 21.6496 | 37.3656 | 28.9579 |
| SIR45 (dB) | 21.0124 | 48.2481 | 26.8540 |
| s1testNovr | 0 | 13* | 0 |
| s1testNadd | 2.71E+03 | 1.8659e+003 | 1.7519e+003 |
| s1testNmult | 2.74E+03 | 2.0339e+003 | 1.8959e+003 |
| s1SNR30 (dB) | 23.2445 | 45.6887 | 25.5719 |
| s1SNR35 (dB) | 22.6308 | 41.8804 | 25.7197 |
| s1SNR40 (dB) | 21.9435 | 43.7642 | 26.1889 |
| s1SNR45 (dB) | 22.026 | 38.5289 | 25.6159 |

Conclusions

Systems 1 and 2 perform similarly to the reference system. Although System 1 has a significantly larger SNR, the difference is inaudible for these inputs. The number of computations for both Systems 1 and 2 were similar and well below the reference system. We suggest System 1 is the best. Its advantage in SNR makes it more robust for a variety of different inputs.

Figure 1 -- ChebychevII Bandpass Filter
Pole-Zero Diagram

Figure 2 -- Elliptic Bandpass Filter
Pole-Zero Diagram